1Exam III and FINAL

Take Home

Fall 2001 Instrumental Analysis

Rules:

1.	You may consult any inanimate object (that is not shared or contain stored information between students) you so desire.

2.	You may consult any dog, cat, parakeet, or other non-human animate object.

3.	You may consult me, or with my written permission, consult another human.

4.	The exam must be written in Word or Word perfect and presented in a printed format.

5.	The due date is Tuesday Dec. 11.  You may turn it in earlier if you so desire.

(25pts) Problem1.

In class we did a “back of the envelope” calculation for the LOD of the ICP-MS.    We found that at a flow rate of 1 mL/min a 1 ppm Co solution (1 ug Co/1 mL= 0.0169x10-6 x6.03x1023 atoms = 1.23x1016 atoms/minute=2.84x1012 atoms/sec) gets ionized.    The number of ions ionized is calculated by the Saha equation and is some fraction of the total atoms based on the ionization potential (Co is 7.86 eV).    In your lecture notes you have a table which show the fraction of Co ionized at 7500K (plasma temperature) as being 93%.    Thus we anticipate that there should be approximately 1012 ions/s moving out of the the plasma.      The atmospheric pressure is dropped in three stages through the ICP-MS from 2 mbar to 10-6mbar in order to remove the Argon gas carrier phase which presents background and in order that each ion within the quadrapole moves with a mean free path that is independent of other ions.    If we assume that our 1012 Co ions are “diluted” by the same fractional pressure drop we arrive at a number of ~ 106 ions/sec moving through the quadrapole to strike the detector for the original 1 ppm solution.

The LOD detection was suggested by the manufacturer of the instrument to be related to the signal obtained in the absence of any solution, determined primarily by the amount of argon ions arriving at the detector.      Argon is present in the absence of any solution as the gas that shapes and modulates the plasma of the ICP-MS.    The signal obtained in the presence of the Argon gas is 5-20±10 cps.

a.)    What is the limit of detection based on the Argon gas background?

b)      Write up a series of steps that my 7th grade son can follow to determine a more realistic LOD.    You do not need to know more instrumental details to answer this question.    You do need to be able to think like an analytical chemist.

(25pts) Problem 2.

In the ASV experiment you found very large background currents which affected the calculated limit of detection.    The peak to peak variation of the background current measured on the blank 30 mL solution was from -0.025 to 0.025 mA centered on 0mA.    40 microliters of 1000 ppm Pb added to 30 mL solution gave a signal measured from 0 mA of 0.0305 mA.    A total of 60 microliters of 1000 ppm Pb standard added to 30 mL solution gave a signal measured from 0 mA of 0.0508 mA.

a.	Given the sample data what is the LOD in terms of signal and in terms of ppm Pb?

b.	The current for lead in ASV film electrode follows the equation:

Define the terms in this equation.

c.	As I see it, you have four obvious options for increasing your signal, ip, for lead which, in turn, will lower your limit of detection.    List the four, choose one, and justify your choice.    Be careful, I may be tricky here!!!    This is one of the few times that I think that you have only one correct answer.    For your answer please estimate the new LOD in terms of ppm Pb.

d.	We have resorted to deconvoluting data in the past (fitting a Gaussian) to the analytical signal to get a signal that is independent of the background.    Would that work for ASV?.    In order to answer this I will want a rational discussion of the physical reality that would make the assumption of a Gaussian (random error) curve for the peak signal correct.

(25pts) Problem 3.

Go to any scientific literature data base and find one example of a separations quantitative (like GC, HPLC, EC) analytical (as opposed to preparative) method for lead.    Get the article and attach it to your exam.

a.	What is the chemistry associated with the separation?    Does it depend upon complexation, if so what makes it selective for lead?    Does it depend upon pH, if so how and in what way?    Does it depend upon phase transition, if so, in what way and how is the transition modulated?

b.	What are the mechanics of the separation?    Packed phase, liquid bonded phase, wall coated?    What is the mobile phase?    What drives the motion of the mobile phase?    What instrumentation is required to drive the motion of the mobile phase?

c.	What parts of the instrument will contribute to peak broadening?    What is the N value for this separation?    Do the authors report it?    Why or why not?

d.	What is the method for detection of the separated lead?    Is there any peak broadening associated with the detection?    Is the detection universal or specific solely to lead?

e.	What is the limit of detection of the method and how did the authors specify the manner in which they calculated it?

f..	What is the selectivity for the method?    Did the authors run a number of different metals to see if they were interferences for lead?

(25pts) Problem 4.

Attached is the class summary data for your soil samples.    The question you proposed to ask was if there was greater lead near the old paint factory as compared to some distance away in park areas.    Perform an analysis of variance to answer this question.    Write a statement that can be sent to the Historical Site manager who approached us about what the heck we were doing.    The statement should give the number of samples taken, the way in which the samples were randomized, the criteria by which you removed samples from the data base, the analytical procedure and quality control.    There should be a table in which you classify samples as “near” and “far” and a statement as to the reason you made that classification.    Maps should be attached.    The data associated with the analysis of variance should be presented in a clear and logical fashion with a written statement.    The statement should indicate what level of certainty you wished to have in your answer.    You may (or may not) wish to compare the entire Pullman area to Chicago background soils.

Should Tuesday data (after analysis) be included in report?fc