1Instrumental Methods of Analysis

Fall 2002

Each part is 11 points.    If you feel that the questions are not understandable and if the procter is unable to answer your questions (ASK!!), feel free to ask to take the exam home for work and email me with questions.    It is never my intention to create exams that are beyond your capacity.

Partial credit is always given

Look at the attached data, shown both in table and in chart recorder form.      At some time after the beginning, t zero, the analyte was injected.

1)	Use twenty points in each calculation.      What are the mean values and standard deviation associated with

I) the baseline (blank) and with

II) the signal?

2)	I) Can you determine if the signal is “real”?

II) Can you quantitfy the signal?

3)	What is the resolution of the analyte from the baseline?

4)	What is the S/N for the data?

5)	From the graph or table estimate the frequency at which the noise is occuring.    What might be the source of the noise?

6)	Draw a circuit which will allow you to reduce the noise amplitude to 0.707 of its initial value.    Indicate the components required to achieve the 0.707 reduction. Show mathematically that this works.

7)	Calculate the change in S/N you expect with your new component.

8)	Estimate the rise time of the instrument.

9)	What range of frequencies have to be measured to adequately track this signal?

.

| First 20 data points |  |  | Last twenty data points |  |  |
| --- | --- | --- | --- | --- | --- |
| rime | signal |  | time | signal |  |
| 0.001 | 4.207967 |  | 0.48 | 2.147799 |  |
| 0.002 | 6.105912 |  | 0.481 | 3.045349 |  |
| 0.003 | 7.427813 |  | 0.482 | 4.638066 |  |
| 0.004 | 7.988203 |  | 0.483 | 6.702484 |  |
| 0.005 | 7.708458 |  | 0.484 | 8.948951 |  |
| 0.006 | 6.62783 |  | 0.485 | 11.06227 |  |
| 0.007 | 4.89794 |  | 0.486 | 12.74594 |  |
| 0.008 | 2.761506 |  | 0.487 | 13.76372 |  |
| 0.009 | 0.518286 |  | 0.488 | 13.97281 |  |
| 0.01 | -1.51698 |  | 0.489 | 13.34388 |  |
| 0.011 | -3.05873 |  | 0.49 | 11.96517 |  |
| 0.012 | -3.89063 |  | 0.491 | 10.03012 |  |
| 0.013 | -3.89598 |  | 0.492 | 7.810228 |  |
| 0.014 | -3.074 |  | 0.493 | 5.616965 |  |
| 0.015 | -1.54004 |  | 0.494 | 3.758056 |  |
| 0.016 | 0.490703 |  | 0.495 | 2.494318 |  |
| 0.017 | 2.733292 |  | 0.496 | 2.003062 |  |
| 0.018 | 4.873087 |  | 0.497 | 2.353215 |  |
| 0.019 | 6.609865 |  | 0.498 | 3.495647 |  |
| 0.02 | 7.699951 |  | 0.499 | 5.270068 |  |
| 0.021 | 7.990403 |  | 0.5 | 7.427515 |  |