1Instrumental Methods of Analysis

Take Exam II

Fall 2003

Rules

You may consult any inanimate object you wish.

You may not consult any animate objects other than your dog/cat/fish, or myself.

You may not exchange discs.

Exam is due one week from pick up during class.

Print out the exam with answers.    Return disc.

Printed version will be the one graded.

All students do problem 1.

8.	Select and answer three of the remaining questions.

8.	My EMAIL: is afitch@luc.edu

ALL STUDENTS DO problem 1.

1.	A.	The maximum measureable absorbance for an instrument with 4.5% stray light is?

B.	If a sample would give a “true” absorbance of “1" on an instrument with 0% stray light what is the measured absorbance for the instrument in “A”?

SELECT THREE OF THE REMAINING PROBLEMS

2. 	Deconvolution

In the attached diskette you will find excel data from graduate student Audrius Zhukas.    The data relates to the fluorescence intensity of 7-hydroxy-4-methylcoumarin (7HC) the “chromophore” of calcein blue as a function of pH.    The fluorescence intensity has been normalized to 1.

a.	(6 pts) Plot the normalized fluorescence intensity of 7HC as a function of wavelength superimposing the three pH values: 0, 5, and 12.5.

Inspect your plot.    You should observe that the fluorescence intensity at pH 0 has a different wavelength maxima than the relative fluorescence intensity at pH 5 and 12.5.      This suggests a different chemical structure is responsible for excitation and emission.    This is similar to dithizone where the two peaks present in the absorbance spectra were attributed to thione and thiol.

b.	(4 pts) Would you expect the excitation wavelength of the species responsible for fluorescence at pH 0 to be shorter or longer than the excitation wavelength for the species responsible for fluorescence at pH 5 and 12.5?

Audrius and I argued about the shape of the relative fluorescence intensity observed at pH 5.    I said that it looked like it was the convolution of two peaks:    one composed of the main species at pH 5 and a smaller one composed of residual fluorescence from the species at pH 0.    That is, that at pH 5 not all of the compound has deprotonated yet.    Audrius said that the peak at pH 5 was composed of a single peak.    Who is right?

c.	(10 pts) Deconvolute the peaks at pH 0, pH 5, and pH 12.5.    Are the peaks best described as a single Gaussian or as composite Gaussians?    Show the deconvoluted spectra and the Gaussians that make up your deconvoluted spectra superimposed onto the raw data.

d.	(5 pts) Discuss the chemical meaning of your results.    I.e.    if the peak at pH 5 consists of two fluorescing species, can you determine the fraction of species deprotonated?    What implications would this have for measuring the Ka value of the compound?

3.	Read the Attached Article

a.	How does matrix change results between the sample and the standard curve?

b.	What is a matrix modifier in general terms?

c.	What is the chemistry that drives matrix effects in this method?

d.	How do the authors demonstrate that their matrix addition method solves the problem that they identified.

4.	Read the 2nd attached article

a.	How was the limit of detection found for this method?

b.	How does the limit of detection compare with other methods for lead analysis?

c.	What is the estimated time for analysis?    What advantages does this method have?

d.	What is the chemistry that gives rise to the signal?

e.	How do the authors “validate” their method?

5&6.	For the compounds shown below one is a colorimetric analytical reagent for metals and the other is a fluorimetric analytical reagent for metals.

Compound “A”		Compound “B”

5.		which is the colorimetric reagent for metals (best analyzed by UV-Vis)?

In the absence of metal

I.	What part of the compound is giving rise to the absorption event?

II.	Is the absorption event likely to be π→π*, σ→σ*, or n→π* , other (define). Be specific, draw the molecule and point to the part that is giving rise to the event.

III.	About what λ would you be looking for the transition?

IV.	If you use a monochromator with a 2000 groves per mm and a    45o angle of incidence, at what angle of reflection will you find your (based on 1.a.III) desired excitation wavelength?

V.	Are solubility and phase changes important parts of the functioning of this compound?

In the presence of metal

VI.	How does the compound “report” the presence of the metal?

VII.	What will be interferences for the measure of, say, lead, by this reagent?

6.	.	Which compound is the fluorimetric reagent for metals?

In the absence of a metal

I.	Which part of the compound is giving rise to the absorption event?

II.	Is the absorption event likely to be π→π*, σ→σ*, or n→π* .or other (define).    Be specific, draw the molecule and point to the part that is giving rise to the event.

III.	About what λ would you be looking for the transition?

IV.	If you use an excitation monochromator with 1500 grooves/mm grating and a 45o angle of incidence at what angle of reflection will the exiting beam need to be to excite your molecule?    (Use your answer B.III.).

V.	What other wavelengths are passed through the excitation monochromator?

In the presence of a metal

VI.	Does fluorescence increase or decrease on binding of the metal?

VII.	Why?

VIII.	Will the absorption event in the presence of the metal take place at the same wavelength as in the absence of the metal?

IX.	About what λ would you be looking for fluorescence.    Give me a numerical answer as in shifted red (long) by 123 nm or shifted blue (short) by 123 nm.

X.	Draw me a spectrum of all the wavelengths that might arrive at the detector.

XI.	How will you configure the instrument to collect the emission and why?

XII.	What features of this molecule may contribute to it’s fluorescence, or to its decreased fluorescence?    Write me the equation for its quantum yield and relate each of the parameters to the structure of the molecule.

7.	(25 pts) Design the Soil Sample and Analysis Protocol

We will be sampling soils in the vicinity of Superfund sites associated with White Lead Paint factories in Chicago.    Design a protocol for sampling and analysis.

You should consider the following parameters: estimated concentrations of lead you anticipate in the soils (you will have to actually read some of the material in the Critical review and “lecture” handouts for this),    total grams of soil to be acquired, randomization of the soil spatially (both depth and horizontal distance),    tools for sampling which will not contaminate, ways of preventing cross contamination from sample to sample, ways to ensure that more than one student can sample without contributing non-random error to the measurement process,    removal of artifacts and soil moisture, location of the sample with respect to some fixed site (for future replicability), addition of controls, splitting of the sample for replication, method for cross comparison between student lab technicians, method for calibration of whatever method you chose to do the analysis (chose from methods you have used to date), determination of a prior criteria for removing data (i.e. you will not be allowed to randomly remove data when you have terminated the project.), ways to account for or mitigate the presence of other interfering species (metals??).      Add anything to your protocol which I may not have mentioned above which you think I have forgotten (intentionally or not).    Explain each step of your protocol.

I do not expect you to deal with the issue of converting the soil to liquid sample yet.    Assume that you can wave a magic wand to carry out this step.

May the Best Protocol Win!!!!